Lab equipments:

DC ANALYSIS
1) Keysight E36313A triple channel programmable dc power supply

1st channel for input current (up to 10A)
	2nd channel for 24 VDC voltage for nonisolated converter, 24 to 5V, for VCC of Current Sensor and Diff. OpAmp

2) Keysight 34460A 6 1/2 Digit Multimeter: To measure the Differential output voltage of OpAmp via RJ45

3) Keysight 34460A 6 1/2 Digit Multimeter: To measure Test Point voltages on the PCB by hand ( yellow points)

AC ANALYSIS

- Keysight E36313A triple channel programmable dc power supply
-> 1st channel for 24 VDC voltage for non-isolated converter, 24 to 5V, for VCC of Current

- Spitzenberger APS 1000 – CC mode : to amplify current in a sine wave shape with varying amplitudes and frequencies. Therefore we achieve AC ripple current with varying frequencies and amplitudes in addition to DC current.

- Wideband Injection Transformer B-WIT 100 : injects differential-mode noise to test the diff. OpAmp circuit susceptibility to noise that affects the difference between two signal lines. Isolated output is differential voltage outputs from the diff. OpAmp.

- Vector Network Analyzer – Bode 100 : to measure impedance by connecting voltage probe to CH2 (with B-WIT 100) and current probe Micsig CP1003B to CH1.

Output channel to AC input modulator’s input channel.

It measures Gain and transforms it directly to impedance Z using: Z=VI=VCH2VCH1=Gain

- Tektronix TBS 2000 Digital Oscilloscope :

-> 1st channel (yellow): differential voltage measurement of diff. OpAmp V_out (V_in of analog card) with B-WIT 100

-> 2nd channel (blue): input current measurement (current probe: Micsig CP1003B)

-> 3rd channel (red): input current measurement (current probe: Tektronix TCP0020)

DC measurements for phase 1-2, with and withput R2 and R5 resistors. Upon comparison, decided to remove them (replaced with 0 ohm resistors) . 
After testing pins,

Current Sensing PCB changes

-Vdd 12V

- voltage rails to be -0.2V (LMM05) and +5.2V using switched regulator and a LDO

- drive common mode voltage of diff op amp to 2.5V (generated by LDO)

-

phase D is broken, we continue with ABC phases

AC evaluation: AC current amplifier, sine wave amp:4 A_p, offset 0A, f = 10 hz -> math red curve (Vp-Vn)  = 100 mV

1-Connect the current output of the Sitzenberger to the input of the current measurement PCB
2-Connect the output of the Bode 100 to the B-WIT transformer’s primary side (allows the Bode 100 to inject a small AC signal combined with the current signal generated by Sipitzenberger.

3- Connect the input CH1 of the Bode 100 in series with the current path, so input of the current sensor. Connect CH2 across the differential outputs of the OpAmp, to measure the voltage response.

4-Connect the secondary side of the B-WIT transformer to the diff. OpAmp

- For differential input impedance, differential inputs of the OpAmp

- For differential output impedance, differential output of the OpAmp

5- Use Bode 100’s oscilloscope to monitor the voltage across the differential outputs of OpAmp (different measurement capabilities of the Bode 100)

6- Configure Bode 100 to perform a frequency sweep. Set the analzyer to measure the gain and phase response of the differential output relative to the injected signal. It measures Gain and transforms it directly to impedance Z using: Z=VI=VCH2VCH1=Gain